German-Russian Institute of Advanced Technologies TU-Ilmenau (Germany) and KNTRU-KAI (Kazan, Russia)

## Guidelines for laboratory work ¼1 of the subject

«Computer systems»

«AVR Studio Simulator Introduction and Exercises»

Kazan 2014

# AVR Studio Simulator Introduction and Exercises

## Introduction

The AVR Studio 4 is an Integrated Development Environment for debugging AVR software. The AVR Studio allows chip simulation and in-circuit emulation for the AVR family of microcontrollers. The user interface is specially designed to be easy to use and to give complete information overview. The AVR uses the same user interface for both simulation and emulation providing a fast learning curve.

Figure 1. AVR Studio learning curve.

## Creating a Project

Once AVR Studio is running you should be prompted for a project to work with, either by creating a new one, or by using an existing project. In this case, click New Project.

Select Project type AVR GCC, enter Project name simcounter, select both Create initial file and Create folder and select (and remember!) a directory in which to put the new simcounter project directory, then click Next.

Select Debug Platform AVR Simulator, and Device ATmega16, then click

## Finish.

You should now see the default window layout in AVR Studio. Because you checked Create initial file when creating the project, AVR Studio has created a blank simcounter.c file and pre-opened this file for you.

If you click on the expand-group icon (the Plus icon) in front of Source Files in the lefthand window, you'll find that simcounter.c has been added to the simcounter project.

If you needed to include any additional files source files for your project, you would right-click on Source Files and choose the appropriate method to do this. You won't need to for this example, but you will for labs later in the semester.

At this point, cut-and-paste the contents of simcounter.c from the final section of this document into the blank window created for your project's simcounter.c. While you're at it, save the contents of this file by doing a File-

>Save (or typing CTRL-S, or pressing the floppy disk icon).

If you had a copy of simcounter.c already on your computer, you could also have used Add Existing File and pointed it to the file's location to add it to the project's source files.

Create a new file called simcounter-PORTA.sti by right-clicking on Other Files in the left-hand window and selecting Create New File. Cut-and-paste the contents of this file from the simcounter-PORTA.sti section at the bottom of this document into the window that was created for this file, and similarly save it to disk.

## Configuring and Building Project

In Project->Configuration Options->General, set Frequency to 8000000 and Optimization to -O0 (note that this is capital-oh zero). It should be noted that this optimization setting is usually not the most popular for generating code to be downloaded to a real microcontroller, but it seems to be a safe choice for code to be simulated. In the future, once you're ready to start programming your microcontollers in the lab, you'll often be directed to change this setting -O2 as you'll see in subsequent

weeks. Should you need to revert to simulating code again generating code  for a real processor, don't forget to reset this option back to -O0. If you ever start seeing strange behavior when stepping execution through simulated programs, it's probable that you weren't using -O0 for the code to be simulated. What usually happens is that while stepping through your program, the cursor doesn't accurately advance from line to line correctly.

After changing this optimization setting to -O0, do a Build->Rebuild All, and you should be all set.

Please note that doing a Rebuild All rather than just a Build is always a good idea after you have changed any project configuration options that have to do with compilation. In fact, it's always safe(st) to do a Rebuild All rather than just a Build

- it's just that it may take somewhat longer.

In Debug->Select device and debug platform, set Debug Platform to AVR Simulator and Device to Atmega16, then click Finish.

Compile the simcounter project with Build->Build (or F7). When the build is complete, confirm that the last line in the Build window at the bottom of the screen

is Build succeeded with 0 Warnings... as shown above. If this isn't what you see, somehow you boffed your cut-and-paste, so go back and try it again.

For some reason, there are certain configuration settings in AVR Studio that cannot be set unless you've already started simulating a program, and the following is one of them. To do so, initiate the simulation with Debug->Start Debugging. Now in Debug->AVR Simulator Options you should set Frequency to 8.00MHz. Go figure..

## Starting and Stepping through Execution

If you hadn't just done so immediately above, normally you'd start simulating the execution of the program with Debug->Start Debugging, or by pressing the green right-facing triangle in the toolbar.

The first thing that the simulator then does is to start-up and then stop before the very

first simulated instruction is executed. It stops here to give you the chance to do all the setup you need before starting the run.

The important thing to notice at this point is that there should be a little yellow right-facing arrow pointing to the opening curly-brace of the function main(). This arrow indicates the next statement to be executed.

At any point when you have control of the running program (that is, when it isn't free-running at fullspeed uninterrupted), you can step through the program slowly, line-by-line, at your own pace. There are three ways to step – step into, step over, and step out. Step into and step over behave similarly except when a function call is the next statement to be executed. Step into steps into (imagine that!) the function and pauses execution at the beginning of the called function. Step over executes the entire function as a single statement and advances to the next statement after the function call. Step over is a huge time-saver once you know that you don't care about stepping through that function line-by-line.

However if you want/need to see what's going on in the function, by all means step into it. Step out is what's used once you're in a function and you no longer care about what's going on inside the rest of it and just want to get back to the code that called the function. Step Over completes executing the function and gets you back to where you were when you called it.

Just to get the hang of single-stepping, we're going to start with Step Into.  You can do this (and most of the simulator functions) one of three ways. In decreasing order of pain, they are: in the menu bar do Debug->Step Into, in the toolbar click on the Step Into icon (the one immediately to the right of the yellow arrow in the toolbar, or by pressing the F11 key. Learn to know and love the F11 (and F10, which is coming-up later) key, as the function keys are by far the easiest ones to use 500 times in an hour.

Press the F11 key once. It should now point to the statement where

buttonPresses is intialiized.

Note again that this is the next statement to be executed. Press the F11 key again and you should see the arrow pointing to uint8_t thisState,  prevState=0x00, changed;

At this point, wouldn't it be cool to know that the value of buttonPresses is actually zero? Well, you can, and there are two ways to do this. The first way is to use your cursor to point to the variable buttonPresses anywhere in the source window and it'll pop-up a floating balloon that says that the current value is 0 (along with some other details about the variable). The second way is the power- user way, and you're also going to learn that now.

You can maintain a window on the screen containing any variables that you want to watch along with their current values. This window is called the Watch window, and you can open it by doing a View- >Watch. Do this and right-click the first cell under the label Name and click Add Item. If you enter buttonPresses, it will give you an entry showing the current name and value for this variable.

Not only that, this value will be updated as you execute the program under your control (it isn't updated as the program free-runs though). An alternate way to add a new variable to be watched is to doubleclick in an empty Name field cell and you can enter a variable name that way. If that weren't enough, there's third way to add a variable to the watch window – just right-click on the variable name anywhere the editing window and select Add Watch.

While you are here, use any of these three methods to add watch entries for the variables thisState, prevState and changed. You should notice at this point in

program execution that we have not yet initialized these three variables. If you  look where the yellow arrow is pointing, we're still before their initialization statements. They are currently what's known as “initialized to garbage”. This is programmer-speak for a variable that has been declared but not yet set (initialized) to a known value – it contains whatever garbage was there when that particular memory location was last used. It is a very, very, very bad thing to ever use a value that has not been initialized. Multibillion dollar satellites (and perhaps your job with it) can be lost forever when this happens, so it's usually best to try to avoid it.

So, observe that these three variables currently contain some random, likely non-zero, value. Then go ahead an press F11 again to execute their initialization statements. Zero now for all of them!

Before you go any farther you're going to learn how to view the I/O registers for the microcontroller.

All PORTA-related registers can be viewed by expanding the PORTA group (click the plus-sign) in the right-hand I/O View window, as well as for all of the other functional groupings of I/O registers. Here you will find the current

simulated values for PORTA, DDRA and PINA both there as well as in their own more-detailed-view window below. Go ahead and expand the PORTB set also. Now if you can't see the full contents of each of these entries' columns, especially in the lower of the two windows, stretch your windows out so that you can see everything. In particular, you want to be able to see all eight squares in the Bits column for each of the three PORTA/PORTB-related registers. These types of

registers are very important as they are the chip's interface to the outside world.

At this point, program execution should be right before the function call to init(). Let's demonstrate Step Into first. The first time that you hit F11 you'll find yourself entering init(). As you hit F11 to execute the following two lines one at a time,

## DDRA = 0x00; // PORTA used entirely for input DDRB = 0xff; // PORTB used entirely for output

You should notice that the values for DDRA and DDRB (also known as Port A/B Data direction register) in the I/O View window are updated. Especially note that the hollow squares in the lower window are blackened for bits that contain high (1) values. This representation is very useful for quickly visualizing which bits are set within an 8-bit value. In the case of DDRB, all bits are high/black – assuming that you have the PORTB group selected in the upper window. You will often be using these windows frequently when following your code's execution.

At this point you should be done executing the last statement in init() and ready to return. Using F11 will return you to the next statement after the one that called this function.

Since we've missed our only opportunity to demonstrate the use of Step Out (because there's only one function call in our demonstration program, and we're never calling it again), let's re-run the program from the beginning. You'll do this often when you get to a point where you've missed something that you mindlessly blew-over and need to start over again. Unfortunately, few simulators are able to backup through a simulation, so you'll have to start at the beginning again.

To halt execution and start over again, press the Stop icon in the menu bar, the blue square (or Debug->Stop Debugging). To restart, as before, press the green triangle (essentially, start) to continue and halt at the first executable statement. Press F11 until you get just inside init(). Rather than stepping through the rest of the function that you no longer care about, you can use Step Out to leave this function. As usual, you can use the Step Out icon to do this, Debug->Step Out, or the appropriate function key.

To demonstrate the final stepping control, do a Debug->Stop Debugging, then Debug->Start Debugging to start the program again, and F11 until you are just about to call init() again. Now use Step Over to execute init() without bothering to step into it. You'll do this often once you are comfortable with what a function does and no longer care about crawling through it again. Stepping into functions that you don't care about is often a huge waste of time. Even better, notice that F10 is the keyboard shortcut for Step Over.

F10 is even more useful than F11 in many circumstances. To be honest, I never know remember which one is which. I'll either use the menu or the toolbar to pick which one that I want the first time, look at the FunctionKey label that is displayed, then just use the FunctionKey from that point on. Just trust me and use the keys – repetitive-stress injuries aren't pretty and every mouse click adds up, trust me.

By the way, one thing that's easy to do is to keep on pressing F11 and then end up in a function you didn't intend to enter. In that case, just do a Step Out and you're back to where you wanted to be.

In summary, those are the three flavors of single-stepping and how each of them are useful, or more efficient, depending on how deliberate you need to be when stepping through code.

## Viewing/changing Variables' and I/O Registers' State

At this point you should have just entered the while loop. This loop is coded to run forever until PINA is read and has the value PINA_END_VALUE (all buttons pressed), at which time it breaks out of the loop. Otherwise, every time through the loop the current contents of PINA are compared to the previous value to see if any bits have changed, indicating that a button has changed state. Press F10 a couple dozen times and see if you can figure out what the program is doing so far.

It should be eventually become obvious that nothing's going to change because, well, nothing's changing. Clearly there aren't any buttons connected our simulated ATmega16 on PINA/PORTA, and until that happens, you'll be stuck only executing the boring part of the loop forever. However, a key feature of the simulator is that you can manually change values on-the-fly to get the program to behave like physical input changes have occurred on the chip's digital inputs.

First, let's change the variable thisState which contains a copy of the current contents of PINA.

Open the Watch window that was introduced before if it's no longer around. If the variable thisState is no longer being watched for some reason, add it again as was done above. Single-step with F10 until you get to the following statement:

## if (thisState == PINA_END_VALUE)

So far it's been the case that thisState has always been zero, causing  you to do little in the rest of the loop and to return to the top again. However, let's force a change. Use the Watch window to change the value of thisState from 0x00 to 0x01. You do this by double-clicking on the 0x00 value and entering 0x01 in its

place (and pressing ENTER/RETURN). Now, with a non-zero value for thisState at this point, pressing F10 should eventually get us into the if block that we've never been in before. The first press of F10 should get you past the break statement, as usual.

Another F10 will cause changed to be set to the difference between the current value and the value from the previous time through the loop. Changed becomes 0x01 because the lower bit has changed since the last time, which then gets you to the inner for loop. By doing enough F10s and following along carefully, you should see that a button press is detected, and buttonPresses is incremented to 1.

If you continue F10-ing enough you'll find yourself back in the outer loop. Assuming that you don't manually alter the value of thisState as we did the last time, PINA will be read again as 0, transferred into thisState, and it'll look like the low bit has changed back to 0, signifying that the button has been released. You'll head into the inner for loop, nothing will appear as a new key press, and eventually you'll be back to where you started, everything zero, nothing changing, looping in the outer part of the while loop forever.

Interesting things happened along the way though. buttonPresses is now set to one because we've “seen” a button press. And if you look in the I/O View window, you should notice that the value of PORTB has changed to reflect the running count of presses detected. If you click on PORTB in the upper window, you'll see the detailed view of the PORTB-related registers in the lower window. Since PORTB would be connected to LEDs in our “real” hardware, one of them would now be lit-up, just like the blackened-square in the lower-right window, in the 0x01 bit position.

Another important feature in addition to being able to change variables' values on-the-fly is that you can also change the I/O registers' values on-the-fly. Just as

you manually changed thisState to modify the captured value of PINA, you can modify PINA directly in a similar fashion. Go ahead and click on PORTA in the upper I/O View window. PINA's bits in the lower window should all be zero –

clear squares in the Bits display. Now click on one of the clear squares for PINA, and you'll see it switch to black. You've turned this bit on manually, and you'll notice that the displayed numerical value changes with it, highlighted in red. This red high-lighting is very helpful feature also, and it's used to point out what value was most recently changed on the display.

Go ahead and press on all these squares to light them all up except one. This would denote that all buttons except one are currently being pressed/held at the same time. Now if you go back and step though the while and for loops, you'll see these additional (fake) button presses counted, eventually resulting in a buttonPresses value of 8, and the binary representation for eight displayed on PORTB.

And just as you can simulate button presses, you can simulate releases by clearing the bits in the display for PINA also. If you set all eight PINA bits at the same time, that's the magic sequence that causes the program to break-out of the outermost while loop and essentially end the program. All buttons pressed results in PINA being 0xFF, to which we've associated the constant label PINA_END_VALUE.

So in summary, you've seen in this section that you're able to modify C- language variables on-the-fly in the Watch window, and able to modify PORT input pins on-the-fly, and can view their output state in the I/O View window.

## TASKS

  - Modify the program so that it acts as a “dead man's switch”, that is, that an alarm sounds whenever a button is not being pressed. Assume that an alarm is connected to the low pin of PORTC and that it sounds when you drive that line's output high.

  - Modify the program to detect when a button is pressed when one are more buttons are already down, and sound the alarm when this occurs.

  - Use the existing stimulus file to drive the same sequence of LEDs flashing as output on PORTB. When any button is pressed on PORTA, the corresponding LED should be lit for the duration of the press, and turned-off when released.

## CONTROL QUESTIONS

- How to create a new project?

- How to build a project?

- How to configure a project?

- How to debug a project?

# SUPPLEMENTAL FILES

## simcounter.c:

/*

- simcounter.c

*

- This program is primarily intended to be used in the AVR Studio

- Simulator for demonstration of debugging features.

- This program counts button presses on eight buttons on PORTA,

- and continuously displays the current count on eight LEDs

- on PORTB.

*

- By going through the exercises that are paired with this program,

- you will become familiar with the AVR Studio Simulator and

- Debugger.

*/

#include <avr/io.h>

Eric B. Wertz 2010/02/04 (v0.02)

SJSU ME106

#define PINA_END_VALUE 0xFF void

init()

{

DDRA = 0x00; // PORTA used entirely for input DDRB = 0xff; // PORTB used entirely for output

}

int main()

{

int buttonPresses=0;

uint8_t thisState=0x00, prevState=0x00, changed=0x00; init();

while (1) { thisState = PINA;

if (thisState == PINA_END_VALUE)

break;

changed = thisState ^ prevState; if (changed != 0) {

for (int i=0; i<8; i++) { uint8_t theBit = (1 << i);

if ((changed & theBit) && (thisState & theBit)) { buttonPresses++;

PORTB = buttonPresses;

}

}

}

prevState = thisState;

}

PORTB = buttonPresses; // just a place to put a final breakpoint while (1)

;

/*NOTREACHED*/

}

## simcounter-PORTA.sti:

00000000:00

00010000:01

Eric B. Wertz 2010/02/04 (v0.02) SJSU ME106

00011000:00

00012000:01

00013000:00

00014000:01

00015000:03

00016000:01

00017000:03

00018000:01

00019000:00

00020000:0F

00021000:00

00022000:03

00023000:02

00024000:01

00025000:00

00100000:FF

99999999:00

## REPORT FORM

German-Russian Institute of Advanced Technologies TU-Ilmenau (Germany) and KNTRU-KAI (Kazan, Russia)

## Laboratory work 1

«AVR Studio Simulator Introduction and Exercises»

Student:

Teacher:

Kazan 2014